- FAQ 四：Notes for pu foam during construction.

- the fill volume：The foaming agent has a certain posterior effect to ensure that the gap is fully filled and therefore there is no need to fill the gap. The amount of filling depends on the product, generally only half of the gap can be filled.The foaming agent has a certain posterior effect to ensure that the gap is fully filled and therefore there is no need to fill the gap. The amount of filling depends on the product, Generally only need to fill half of the gap.

- Filling method：The vertical gap should be horizontal or slightly downward, fill from bottom to top to prevent the foam from flowing upside down; the horizontal gap should be horizontal or a little downward, filled from left to right, of course, individual operating habits can have different Differences; The filling of extra large gaps should not be filled at one time. The construction should be carried out in several stages. The second construction should be carried out after the last time the foam is filled.

- 3) When filling a confined space, ensure that there is sufficient moisture in the cavity and perform the batch construction.

- 4) It is recommended that the one bottle of pu foam should be used up at one time. If there is residual material in the can after the construction is completed, the foam gun can be left on the can and the flow adjustment valve can be closed so that the can  stand in a cool and shady place. It can be used every other day.

- 5) During the construction process, if there is liquid splashed on the body clothes, it can be scraped clean and cleaned with a cleaner; or after the foam is dried, it can be peeled off by hand. The liquid splashed on the skin should immediately be wiped dry with a cloth, washed with soap, or cleaned with a cleaner.

- 6) During the construction process, construction workers are not allowed to smoke; the product is not allowed to approach the open flame, and empty cans that are used up must not be burned or punctured and must not be given to children to play.